VEC Java Security Update Instructions

- Follow the instructions on upgrading the application.

- Open Windows Explorer and navigate to the folder C:\VEC.

- Open another Windows Explorer session.  Click on the address field and enter the following text (without quotes) “ftp://<place unit IP address here>.   Enter the administrator level Username and password when prompted with the FTP logon window.

- When logged in correctly, window should appear as follows.   As an extra security measure, the “java” folder is only listed for administrator level users.

- Double click the folder labeled “java”.

- One by one, drag and drop the files “vec.jar”, “apPopupMenu.jar”, and “zoom.jar” from the Windows Explorer session opened in #2 into the FTP window.

- Close both Windows Explorer sessions.

Clear out Java cache:

- Go to Windows Control Panel and open the Java Control Panel application.

- By default, the General Tab should be selected.  Select it if it is not.

- Click on the Settings button under “Temporary Internet Files”.

- Click the “Delete Files…” button and make sure box for “Cached Applications and Applets” is checked.  Select OK to delete the files.

- Click Cancel and then OK to exit Java Control Panel.

Login after upgrade:

- Open Internet Explorer and navigate to the Power System.  When prompted with the Java Security window, click the check box and then click “Run” button.  This security warning is standard and will only be issued the first time the Power System is accessed after the upgrade.

- When the following minor Java Security warning is issued, just click the “Run” button to allow the application to execute.